NTA8A01 Module Description:

Working voltage: 5V version: DC 3.7-5.5V; 12V version: 6-25V

Working current: 7-13MA

MODBUS RTU protocol, 03 read command, 06 write command.

Serial port baud rate: 9600 (default), N, 8, 1

By modifying the 485 address, up to 247 modules can be cascaded (more than 16 please use R485 repeater)

Adapted sensor: B3950 10K 1% NTC thermistor

Temperature measurement range (using our NTC sensor): -20 ° C to +125 ° C.

Temperature measurement accuracy (using our NTC sensor): 1%.

Size: 30 X 15 X 6MM

Weight: 5V version 2g, 12V version 2.5g

MODBUS RTU protocol  please refer to : " NTA8A01 modbus rtu protocol"

NTC sensor specifications:

Product Name: 5*25mmNTC Waterproof Probe

Total length : 50CM

Resistance value: 10K

Resistance accuracy: ±1%

Resistance B value: 3950 ± 1%

Temperature range: -20 ° C -125 ° C

Wire size: 2651 26# parallel line temperature 105 °C

Plug model: XH2.54-2P

Probe: A version without mounting holes, B version with mounting holes (4MM/M4)

Note:

1 This module does not have an NTC sensor. The NTC sensor needs to be purchased separately (B3950 10K 1%);The module's measured resistance value is 1%, and the theoretical temperature range is -273-200°C. The final temperature measurement accuracy and temperature range are determined by NTC thermistor.

2 Be sure to use the correct sensor (B3950 10K 1% NTC), otherwise the temperature will not be accurate

Wiring diagram: