Product Name:  DC 3.7-25V DS18B20 RS485 RS232 TTL Modbus Rtu Temperature Sensor Remote acquisition monitor Digital Thermometer Module

Packing list:

1 pcs R46CA01 RS485/TTL232  Temperature sensor Board(Please choose the version you need)

Description :

Working  voltage :A/B 5V Version : DC 3.7-5.5V; C 12V Version : DC 6-25V
  
Working  current:5-8MA
  
NO : no DS18B20 temperature sensor; TS : with DS18B20 temperature sensor
  
A: 5V version TTL232 interface;B: 5V version RS485 interface; C: 12V version RS485 interface
  
MODBUS RTU protocol,03 Read,06Write.
  
Serial baud rate:9600（default）,N,8,1
  
By modifying the RS485 address, up to 247 module s can be connected (more than 16 US ether485 repeater)
 
Temperature sensor is DS18B20
  
Measures Temperatures from -55°C to +125°C(-67°F to +257°F)
  
±0.5°C Accuracy from -10°C to +85°C
  
Size:A/B Version 30X15X3.5mm; C Version 30X15X6mm
Weight:no DS18B20 1.4-1.9g,with DS18B20 1.9-2.2g

For more information, please contact us!!!

Typical application:

Thermostatic Controls
Industrial Systems
Consumer Products
Thermometers
Thermally Sensitive Systems
Indoor and outdoor temperature measurement
Temperature in vegetable garden
Weather forecasting, and monitoring
Temperature in Computer Room
Warehouse temperature