WG18A02 RS485 Weighing Module Manual

DEMO : https://youtu.be/fDDWNzxewwg

Function description:

1 Working voltage: 5V (5V version), 8-16V (12V version);

2 Working current: 12-15MA;

3 MODBUS RTU command supports 03, 06, 16 function codes;

4 RS485 interface/TTL232 interface optional

5 Sensor model: HX711, 24-bitADC

6 Load cell input signal range: ±20mv

7 Sampling rate: 10HZ or 40HZ can be set

8 The functions of baud rate, division value, zero-chasing range, median filter, average filter, dynamic tracking, etc. are integrated inside.

9 Default baud rate 9600 N 8 1, baud rate and parity bit optional

10 If the setting is wrong, you can short-circuit "RESET" for 5 seconds to restore the factory settings

11 In MODBUS RTU command mode, up to 247 devices can be used in parallel

12 size: 45*16.5*6.2mm

13 Weight: 4 grams

For Modbus rtu command details, see "WG18A02 Modbus RTU Commands"

For software usage, please refer to "WG18A02TestTool Instructions for Use"

Wiring diagram:

Pin description:

Weight Sensor

Working voltage: DC 5-10V

Working current: 200UA

Measuring range: 0-1KG, 0-5KG, 0-10KG, 0-20KG

Size: 80*12.7*12.7mm

Weight: 30 grams

Pins: Red: Power +, Black: Power -, Green: Output Signal +, White: Output Signal -

Note: One end of the sensor is fixed through the screw hole, and the other end is kept in a suspended state, and gravity is applied in the direction indicated by the label. Special attention must be paid not to press the white glue-coated part directly, so as not to damage the sensor.