N4DBA06 8-channel analog digital IO module

Features:

1Working voltage: DC 12 (DC 12-25V recommended power supply with small ripple)

2 Working current: 14-34MA (Related to the output current)

3 MODBUS RTU instruction support 03 06 function code

4 3 analog input channels, 1 switch input channel: Vi1 is 0-5V voltage input, Vi2 is 0-10V voltage input, Ii is 0-20MA current input; DiP DiN is switch input port (NPN/PNP Photoelectric isolation input)

5 3 analog output channels, 1 switch output channel: Vo1 is 0.05-5V voltage output (<20MA), Vo2 is 0.1-10V voltage output (<20MA), Io is 0.5-20MA current output; DoP DoN is Switch output port (maximum drive current 500MA, can directly drive relays and solenoid valves)

6 Voltage resolution is 0.01V, measurement accuracy is 1%; Current resolution is 0.01MA, measurement accuracy is 1%; if the error is greater than 1%, it can be calibrated

7 MODBUS commands can be made serial HyperTerminal (serial assistant)  OR PLC Enter;

8 Under the MODBUS command mode, it can support up to 247 devices in parallel

9 Size: 60 * 72 * 18mm (Only PCB Board); 63 * 87 * 42mm (with Din Rail Box)

10 Weight: 33g (Only PCB Board); 85g (with Din Rail Box

)

Modbus RTU Command Please refer to : " N4DBA06 modbus rtu protocol"

DIN rail Box parameters:

Product model: UM72

Color: green

Width: suitable for PCB board width UM72(72mm)

Insulation grade: flame-retardant VO grade

Backplane length: suitable for 60 mm PCB boards

Net weight: 52g

Installation: DIN35 and C45 rail

Note:

1 Channel 1 (Vo1) output voltage range is 0.05V-5V, channel 2 (Vo2) output voltage range is 0.1V-10V, channel 3 (Io) output current range is 0.5-20MA

2 At the moment of power-on, the current and voltage output ports have 10-20MS high pulses

3 DiP DiN is a passive switch input port (see the figure below for wiring)

4 DoP DoN is a passive switch output port (see the figure below for wiring)

Slave ID: Different "Sliver ID" can be set by command, the maximum number is 247

Under the MODBUS command mode,the slave ID must be correct

Command Description, Please refer to " N4DBA06modbus rtu protocol "