SHA8C01 RS485 SHT30 sensor Instructions

SHA8C01 RS485 SHT30 Acquisition module Description:

Working voltage: 5V version: DC 3.7-5.5V; 12V version: 6-25V

Working current: 7-13MA

MODBUS RTU protocol, 03 read command, 06 write command.

Serial port baud rate: 9600 (default), N, 8, 1

By modifying the 485 address, up to 247 modules can be cascaded (more than 16 please use R485 repeater)

Adapted sensor: SHT30 temperature and humidity sensor

Temperature measurement range: -40-125℃ (Because the shell and wire are not resistant to high temperature, please use it in the environment of -40-80℃)

Humidity measurement range: 0-100%RH Accuracy: ±0.2℃; ±2%RH

Size: 30 X 15 X 5.8MM

Weight: 2g

MODBUS RTU protocol  please refer to : " SHA8C01 RS485 SHT30 sensor protocol "

SHT30 sensor specifications:

Product model: TB446 SHT30

Working voltage: 2.5-5.5V DC

Output signal: IIC digital signal

Output interface: XH2.54 4P terminal block

Temperature measurement range: -40-125℃ (Because the shell and wire are not resistant to high temperature, please use it in the environment of -40-80℃)

Humidity measurement range: 0-100%RH

Accuracy: ±0.2℃; ±2%RH

IIC address: 0X44

Wire length: 30 cm

Weight: 15 grams

Shell material: ABS

Wiring diagram:

RS485 bus wiring diagram:

SHT30 sensor: